[image: image1.png]Distributed by

£

1208 North Grand * Roswell, NM 88201 {
(505) 623-9726

Pere
LR
S

- i MSDS Number:‘22i‘85‘j‘
HUNTSMAN Material Safety Data Sheet Revised: OQ-NOV—ZOOQM'E

Polyurethanes

/.
%

EMERGENCY CONTACTS

Spilis, Leaks, Fire or Exposure Call Chemtrec: (800) 424-9300
Medical Emergency Information: (800) 328-8501

SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Product Name: RUBINATE® M
Product Use: Component of 2 polyurethane.
Company:

Huntsman Polyurethanes (a business unit of Huntsman /C} Chemicals LLC.)
286 Mantua Grove Road
West Deptford, NJ 08066-1723

For Polyurethanes product information/assistance:
West Deptford: (800)257-5547 Auburn Hills: (800)553-8624
Canadian Office: (905)678-9150

SECTION 2 - COMPOSITION / INFORMATION ON INGREDIENTS

HAZARDOUS INGREDIENT(S) % (wiw) ACGIH TLV CAS NO.
Polymeric Diphenylmethane - 100 Not Listed 8016-87-9
Diisocyanate (polymeric MDI)

CONTAINS:

4,4'-Diphenyimethane-

Diisocyanate (4,4 MDI) 0.005 ppm 101-68-8

(approx. 45%)

MDI isomers/oligomers Not Listed 9016-87-9

This material is classified as hazardous under OSHA Hazard Communication Standarc (28 CFR
1910.1200).

SECTION 3 - HAZARDS IDENTIFICATION

AR A A O N R K AR K KOOI ORI KK IR RO IR IR K KR K 00 B8O I I S KR O K KK

EMERGENCY OVERVIEW

Health Hazards: Irritating to eyes, respiratory system and skin. [nhalation at levels above the
occupaticnai exposure fimit could cause respiratory sensitization and risk of serious damage to
respiratory system. The onset of the respirciory symptoms may be delayed for several hours after
exposure. A hyper-reactive response to even minimal concentrations of MD! may develop in
sensitized persons. Sensitized persons should not be exposed to any mixture containing unreacted
MD!.




[image: image2.png]RUBINATE® M ( Revised:09-NOV-2000) Page 2 ‘

Physical Hazards: Reacts slowly with water to produce carbon dioxide which may rupture closed
containers. This reaction accelerates at higher temperatures

Appearance: Dark brown liquid.
Odor: Slightly musty. 4

Read the entire MSDS for 2 mare thorough assessment of the hazard infarmation on this product.
*******i*x)l(****t*t**ﬁn&‘)K‘('ﬂ!‘iil!,**lx**.***#l‘!*lt!t,‘

SECTION 4 - FIRST AID MEASURES

General: In case of accident or if you feel unwell, seex medical advice IMMEDIATELY (show the
label where possible).

Inhalation: Remove patient from exposure, keep warm and at rest. Obtain medical attention.
Treatment is symptomatic for primary irritation or difficulty in breathing. If breathing is labored
oxygen should be administered by qualified personnel. Apply artificial respiration if breathing has
ceased or shows signs of failing.

Skin Contact: Remove contzminated clothing. Immediately wash affected areas thoroughly with
soap and water. Some organic materials such as corn oil or propylene glycol are effective in
decontaminating MDI from the skin when applied immediately. If irritation, redness, or a burning
sensation develops and persists, ottain medical advice. Contaminated clothing should be
tharoughly cleaned before reuse.

Eye Contact: Immediately flush eyes with running water for a minimum of 15 minutes. Held
eyelids open during flushing. If irritation persists, repeat flushing. Obtain medical attention
IMMEDIATELY.

Ingestion: Do NOT induce vomiting. Provided the patient is conscious, wash out mouth with
water then give 1 or 2 glasses of water to drink. Refer person to medical personnel for immediate
attention.

Note to Physicians: Symptomatic and supportive therapy as needed. Folowing severe exposure
medical follow-up should be monitored for at least 48 hours.

SECTION 5 - FIRE-FIGHTING MEASURES

Fire and Explosion Hazards: Containers may burst under intense heat. Dus to reaction with water, a
hazardous build-up of pressure could resuit if contaminated containers are re-seale.

Extinguishing Media: Water, carbon dioxide, dry chernical, or appropriate foam. If water is used,
very large quantities are required. Reaction between water and hot isocyanate may be vigorous.
Contain run-off water with temporary barriers.

Fire Fighting Procedures: As appropriate for surrounding materials/equipment.

Fire Fighting Protective Equipment: Use self-contained breathing apparatus and full protective
clothing (Bunker gear).

Flash Point: 425°F (218°C)

Flammable Limits (Lower): Not available.

Flammable Limits (Upper): Not available.

Auto Ignition Temperature: 240°C (464°F) (4,4'~ Diphenyimethane Diisacyanate)
Decomposition Temperature: Not available.

Rate of Burning: Not available.





[image: image3.png]RUBINATE® M ( Revised:09-NOV-2000) Page 3 ;

Explosive Power: None.

Sensitivity to Mechanical Impact: None.

Sensitivity to Static Discharge: None.

Combustion Products: Carbon monoxide, carbon dioxide, nitrogen oxides and some HCN.. 7,

SECTION 6 - ACCIDENTAL RELEASE MEASURES

For major spilis cal. Chemtrec (800-424-9300).

Spills;Leaks, ar Releases: Clean-up should only be psrformed by trained personnel. People
dealing with major spillages should wear full protective clothing including approgriate respiratory
protection. Evacuate the arez. Prevent further ieakage, spillage or entry into drains.

Contain and atsorb large spillages onto an inert, non-flammable adsortent carrier (such as
earth or sand). Shovel into open-top drums or plastic bags for further decontamination, if
necessary. Wash the spillage area clean with liquid decontaminant. Test atmosphere for MDI.
Neutralize small spillages with decontaminant. Remove and properly dispose of residues. (See
Section 13 for disposal considerations.) Notify applicable government authorities if release is
reportable. The CERCLA RQ for MDI is 5,000 Ibs (see CERCLA in Section 15).

Preparation of Dacontamination Solution: Prepare a decontamination solution of 0.2-0.5%
liquid detergent and 3-8% concentratec ammanium hydroxide in water (5-10% sodium cartonate
may be substituted for the ammonium hydroxide). Follow the precautions on the supplier's
material safety data sheets when preparing and using solution.

Use of Decontamination Solution: Allow deactivated material to stand for at least 30 minutes
before shoveling into drums. Do not tighten the bungs. Mixing with wet eartr is also effective, but
slower.

SECTION 7 - HANDLING AND STORAGE

Handling: Avoid personal contact with the product or reaction mixture. Use only with adequate
ventilation to ensure that the occupational exposure limit is nct exceeded. The efficiency of the
ventilation system must be monitored regularly because of the possibility of blockage. Avoid
breathing aerosols, mists and vapors. (See Section 8—Exposure Contro! for details.)

Storage Requirements: Keep containers properly sealed and when stored indoors, i a well
ventilated area. Keep contents away from moisture. Due to reaction with water, producing
CO2-gas, a hazardous build-up of pressure could resuit if contaminated containers are re-sealed.
Do not reseal contaminated containers. Uncontaminated containers, free of moisture, may be
resealed only after placing under a nitrogen blanket. Do not stcre in containers made of copper,
copper alloys or galvanized surfaces.

Storage Temperature: Ideal storage temperature is 16-38°C (60-1 00°F).
Keep stocks of decontaminant (See Section 6) readily available.

SECTION 8 - EXPOSURE CONTROLS/PERSONAL PROTECTION

PREVENTIVE MEASURES:

Conditions of use, adequacy of engineering or other contro! measures, and actual exposures will dictate the
need for specific protective devices at your workplace.

Engineering Controls: Use local exhaust ventilation to maintain airborne concentrations below




[image: image4.png]RUBINATE® M ( Revised:09-NOV-2000) Paged

the TLV. Suitable respiratory equipment should be used in cases of insufficient ventilation or
where operational procedures demand it. For guidance on engineering control measures refer to
publications such as the ACGIH current edition of "Industrial Ventilation, a manual of
Recommended Practice.” .

Personal Protective Equipment:

Eye Protection: Chemical safety goggles. If there is a potential for spiasning, use a full
face shield.

Skin Protection: The following protective materials are recommended:

Gloves - neoprene, nitrile rubber, butyl rubber. Thin latex disposable gloves should be
avoided for repeated or long term use.

Protective clothing shouid be selected and used in accordance with "Guidelines for the
Selection of Chemical Protective Clothing" published by ACGIH.

Respiratory Protection: When the product is sprayed or heated without adequate
ventilation, an approved MSHA/NIOSH positive-pressure, supplied-air respirator may be
required. Air purifying respirators equipped with organic vapor cartridges and a HEPA
(P100) particulate fiiter may be used under certain conditions when a cartridge change-out
schedule has been developed in accordance with the OSHA respiratory protection standarc
(29 C.F.R. 1910.134).

EXPOSURE GUIDELINES:

Medical supervision of all employees who handle or come in contact with respiratory sensitizers is
recommended. Persons with respiratory problems including asthmatic-type conditions, chronic
bronchitis, other chronic respiratory diseases or recurrent skin eczema or skin allergies should be
evaluated for their suitability of working with this product. Once a person is diagnosed as
sensitized, no further exposure to the material that caused the sensitization should be permitted.

HAZARDOUS INGREDIENT(S):

4,4'-Diphenyimethane Diisocyanate:
ACGIH TLV 0.05 mg/M? (8-hour, 40 hoursiweek)
OSHA PEL CEILING  0.20 mg/Mm®
NIOSH REL/TWA 0.05 mg/M® (10-hour, 40 hours/week)
NIOSH REL/CEILING  0.20 mg/M? (10-minute)

NOTE: The Occupational Exposure Limits listed for isocyanates do not apply to previously
sensitized individuals.

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES

Alternate Name(s): Polymeric MDI

Chemical Name: Not applicable (mixiure).

Chemical Family: Diisocyanate.

Molecular Formula: Not applicable (mixture).

Appearance: Dark trown liquid.

Odor: Slightly musty.

Odor Threshold: 4.0 mg/M® (4,4'-Diphenyirsthane Diisocyanate)
pH: Not applicable,

Flash Point: 425°F (218°C)

Vapor Pressure (mm Hg at 20°C): Approx. 4 x 104
Vapor Density (Air=1); 8.5 approx.




[image: image5.png]RUBINATE® M ( Revised:09-NOV-2000) Page5

Boiling Point: Not applicable.

Melting Point: Not available.

Solubility (Water): (Reacts with water)

Solubility (Other): Soluble in most organic solvents
Specific Gravity: 1.24 approx. (at 25°C)
Evaporation Rate: Not available.

SECTION 10 - STABILITY AND REACTIVITY

Hazardous Decomposition Products: Highly unlikely under normal industrial use. See Section
5.

Chemical Stability: Stable at room temperature.

Conditions to Avoid: Avoid high temperatures. Avoid freezing.

Incompatibility with other Substances: This product will react with any materials containing
active hydrogens such as water, alcohol, amines, bases and acids. The reaction with water is very
slow under 50°C (122°F) but is accelerated at higner temperatures.

Hazardous Polymerization: Polymerization may occur at elevated temperatures in the presence
of alkalies, tertiary amines and metal compounds.

SECTION 11 - TOXICOLOGICAL INFORMATION

Polymeric MDI:

Oral LD50 (rat) > 5,000 mg/kg
Dermal LD50 (rabbit) > 5,000 mg/kg

POTENTIAL HEALTH EFFECTS:

Inhalation: This product is a respiratory irritant and potential respiratory sensitizer. Inhaiation of
vapor or aerosol at levels above the occupational exposure limit could cause respiratory
sensitization and lung injury. Symptoms may include irritation to the eyes, nose, throat and lungs,
possibly combined with dryness of the throat, tightness of chest and difficuity in breathing and/or
flu-like symptoms. The cnset of the respiratory symptoms may be delayed for several hours after
exposure. A hyper-reactive response to even minimal concentrations of MDI may develop in
sensitized persons. In a single evaluation of 5§ men occupationally exposed to MDI and
hydracarbon soivent vapors under conditions where adequate ventitation or other safety
precautions were not used, neuropsychologic findings were attributed to MDI.

Skin Contact: Moderate irritart. Repeated and/or prolonged contact may cause skin sensitization,
There is limited evidence from animal studies that skin contact may play a role in respiratory
sensitization. These results emphasize the need for protective clothing inciuding gloves to be worn
at all times when handling these chemicals or in maintenance work.

Eye Contact: The aerosol, vapor or liquid will irritate human eyes following contact.

Ingestion: Ingestion may cause imitation of the gastrointestinal tract. Based on the acute oral
LD50, this product is considered practically non-toxic by ingestion.

Chronic Effects: A study was conducted wnere groups of rats were exposed for 8 hours/day, 5
days/week for a lifetime to atmespheres of respirable polymeric MDI aerosol at concentrations of 0,
0.2, 1 or 6 mgiM®. No adverse effects were observed at 0.2 mg/M?®. At the 1 mg/M* concentration,
minimal nasal and lung irritant effects were seen. Only at the top concentration (8.0 mg/M?) was
there an increased incidence of a benign tumor of the lung (adenoma). One malignant pulmanary




[image: image6.png]RUBINATE® M ( Revised:09-NOV-2000) Page 6

i

tumor (adenacarcinoma) was seen in tha 6.0 ma/M* group. MDI administration to rats in this study
did not change the distribution and incidence of tumors from those seen in control animals. The
increased incidence of lung tumors is associated with prolonged respiratory irritation and the
concurrent accumulation of yellow material in the lung. In the absence of prolonged exposure to
high concentrations leading to chronic irritation and lung damage, it is highly unlikely that tumo¥
formation will occur.

There are reports that excessive chronic expasure to diisocyanates may result in permanent
decrease in lung functicn.

Carcinogenicity: The ingredients of this product are not classified as carcinogenic by ACGIH or
IARC, not regulated as carcinogens by OSHA, and not listed as carcinogens by NTP.

Mutagenicity: There is no substantial evidence of mutagenic potential.
Reproductive Effects: No adverse reproductive effects are anticipated.

Teratogenicity and Fetotoxicity: No birth defects were seen in two independent animal (rat)
studies. Fetotoxicity was observed at doses that were extremely toxic (including lethal) to the
mother. Fetotoxicity was not observed at doses that were not matemally toxic. The doses used in
these studies were maximal, respirable concentrations well in excess of the defined occupationat
limits,

SECTION 12 - ECOLOGICAL INFORMATION

Environmental Fate and Distribution: It is unlikely that significant environmental exposure in the
air or water will arise, based on consideration of the production and use of the substance.

Persistence and Degradation: Immiscible with water, but will react with water to produce inert
and non-biodegradable solids.

Toxicity: Polymeric MDI.

LC50 (Zebra Fish) > 1000 mg/! (At the highest level tested of 1000 mg/l there were no deaths.)
EC50 (Daphnia magna) (24 hour) > 1000 mg#

EC50 (E. Coli) > 100 mg/t

SECTION 13 - DISPOSAL CONSIDERATIONS

The generation of waste should be avoided or minimized wherever possible.

Disposal should be in accordance with local, state, provincial or naticnal reguiations. This materiat
is not a hazardous waste under RCRA 40 CFR 261, Small quantities should be treated with a
decontaminant solution (See Section 6). The treated waste is nota hazardous material under
RCRA 40 CFR 261. Chemical waste, even small quantities, should never be poured down drains,
sewers or waterways.

Empty containers should be deccntaminated and either passed to an approved drum recycler or
destroyed.

SECTION 14 - TRANSPORT INFORMATION

DOT: Single containers less than 5,000 ibs. are not regulated. Single containers with 5,000 lbs. or
more of 4,4-MDI are regulated as: Other Regulated Substances, Liquid, N.O.S. (Methyiene
Diphenyl Diisocyanate), 9, NA3082, PGIll, RQ.




[image: image7.png]RUBINATE® M ( Revised:09-NOV-2000) Page 7

Transportation Emergency Telephone Number: 1-800-424-8300 (CHEMTREC)
TDG: Not Reguiated.

IMO: Not Regulated.
IATA/ICAO Class: Not regulated.

SECTION 15 - REGULATORY INFORMATION

USA CLASSIFICATION:

OSHA Classification: This product is classified as a hazardous materia! under the criteria outlined
in the OSHA Hazard Communication Standard (HCS) (28 CFR 1910.1200).

TSCA (Toxic Substances Control Act) Regulations: All ingredients are on the TSCA Chemical
Substance Inventory.

EPCRA Section 313 (40 CFR 372): This product contains the following chemical(s) subject to
reporting requirements:  100% Diisocyanate compounds (Category Code N120).

CERCLA (Comprehensive Environmental Response, Compensation and Liability Act):
4,4-Methylene dipheny| diisocyanate (CAS 101-68-8) has a 5,000 Ib. RQ (reportable quantity). Any
spill or release above the RQ must be reported to the National Response Center (800-424-8802).
The % of 4 4-MDI in this product is listed in Section 2 of this MSDS.

This product does not contain nor is it manufactured with czcne depleting substances.

Other Regulations/Legislation which apply to this product: Massachusetts Right-to-Know,
Pennsylvania Right-to-Know, New Jersey Right-to-Know, CERCLA.

CANADIAN CLASSIFICATION:
This product has been classified in accordance with the hazard criteria of the CPR (Contrclied
Froducts Regulations) and this MSDS (Material Safety Data Sheet) contains all the information
required by the CPR.
Controlled Products Regulations (WHMIS) Classification: D-1A; D-2A and D-2B.

CEPA / Canadian Domestic Substances List (DSL): The substance(s) in this product is/are on
the Canadian Domestic Substances List (CEPA DSL)

SECTION 16 - OTHER INFORMATION

Glossary: ACGIH-  American Canference of Governmental Industrial Hygienists

|IARC - International Agency for Research on Cancer
NTP - National Toxicology Program
OSHA - Occupational Safety and Health Administration

For Your Protection: Huntsman Polyurethanes warrants only that its products meet the
specifications stated in the sales contract. Typical properties, where stated. are to be considered as
representative of current production and should not be treated as specifications. While all the
information presented in this document is beneved to be reliable and to represent the best available
data on these products, NO GUARANTY, WARRANTY, OR REPRESENTATION IS MADE,
INTENDED, OR IMPLIED AS TO THE CORRECTNESS, OR SUFFICIENCY OF ANY
INFORMATION, OR AS TO THE MERCHANTABILITY OR SUITABILITY OR FITNESS OF ANY
CHEMICAL COMPOUNDS OR OTHER PRODUCTS FOR ANY PARTICULAR USE OR




[image: image8.png]RUBINATE® M ( Revised:09-NOV-2000) Page 8

PURPOSE, OR THAT ANY CHEMICAL COMPOUNDS OR OTHER PRODUCTS OR THE USE
THEREOF ARE NOT SUBJECT TO A CLAIM BY A THIRD PARTY FOR INFRINGEMENT OF ANY ¥
PATENT OR OTHER INTELLECTUAL PROPERTY RIGHT. THE USER SHOULD CONDUCT
SUFFICIENT INVESTIGATION TO ESTABLISH THE SUITABILITY OF ANY PRODUCT FOR ITS
INTENDED USE. Liability of Huntsman Polyurethanes for all claims, whether arising out cf breach
of warranty, negligence, strict liability, or otherwise, is limited to the purchase price of the material.
Praducts may be toxic and require special precautions in handling. For all products listed, the user
should obtain detaited information on toxicity, together with proper shipping, handling, and storage
procedures, and comply with all applicable safety and environmental standards. Toxicity and risk
characteristics of chemical compounds and other products may differ when used with other
materials or in a manufacturing or other process. Those risk characteristics should be determined
by the user and made known to handlers, processors, and end users.

MATS Number:
MATS Index:

Prepared By: Polyurethanes Applied Toxicology/Regulatory Services (856)423-8484




[image: image9.png]v

MATERIAL SAFETY DATA SHEETS

Uucsc EMERGENCY PHONE #S
1208 N. GRAND Roswell 505-623-9726

ROSWELL, NM 88201 Phoenix 602-269-9711
. CHEMTREC 800-424-9300

SPS II 1.7 (B COMPONENT)

SECTION 1. MATERIAL IDENTIFICATION

Chemical Family: POLYETHER POLYOL

Chemical Name: POLYMERIC ALCOHOL

Synonyms: POLYOL, URETHANE RESIN, B COMPONENT
CAS: N/A - BLEND

SECTION II. INGREDIENTS AND HAZARDS

Ingredient Name: Approx.%  Exposure Limits

Hydroxyl Terminated Poly (Oxyalkylene) Polyether 63-94% N.E.

Tertiary Amine Bearing Compounds <2 N.E.

1,1 Dichloro-1-flucroethane CAS #1717-00-6 3-34 Suggested SO0 ppm
SECTION III. PHYSICAL DATA

Appearance: Viscous Liquid

Color: Amber to Dark Brown

Odor; Slightly Ethereal

Molecular Weight: NA

Melt Point/Freeze Point: <B2F°(«0° Q)

Boiling Point: Decomposes

Vapor Pressure: Not Established

Vapor Density (air=1): Not Established

Specific Gravity (H20=1): 9.2 1bs/gal.

‘Water Solubility: Slight

VOC %: 30%

UCSC LTD: 1208 North Grand * Roswell, New Mexico 88201 » (505) 623-9726 » FAX {505) 623-1908




[image: image10.png]SECTION IV. FIRE AND EXPLOSION DATA

FLASH POINT: 248’ F (120° C) PMCC

Limits - LEL %: Not Established (non-volatile fluid)
UEL %: Not Established (non-volitale fluid)

EXTINGUISHING MEDIA: Dry chemical extinguishers such as monoammonium phosphate,
potassium sulfate, potassium chloride. Additionally, carbon dioxide, high expansion
(proteinic) chemical foam, water spray for large fires.

SPECIAL FIRE FIGHTING PROCEDURES: Do not direct solid water stream or foam into
hot, buming pools; this may cause frothing and increase fire intensity. Use self-contained
breathing apparatus and body covering protective clothing; burning can produce oxides of
carbon and nitrogen.

UNUSUAL FIRE AND EXPLOSION HAZARDS: Contact with certain finely devided
reactive materials may cause reactions. Decomposition products may be hazardous.

SECTION V. HEALTH EFFECTS DATA
ANIMAL TOXICITY:

Oral, LD50 (ingestion) NE
Dermal, LD50 (skin contact) NE
Inhalation, LC50 (4 hrs.) NE
Aquatic, LC50 (24 hrs.) NE
Eyes NE
Skin NE

HUMAN EFFECTS OF OVEREXPOSURE:

INHALATION: May cause irritation to the throat and respiratory passages but at room
temgcrature. vapor inhalation is not considered hazardous. This product contains HCFC
141B. Atelevated temperatures, 141B gas may escape. Vapors may cause dizziness, tremors,
cardiac arrythmias and cardiac arrest.

SKIN: This product contains amine catalyst and will cause irritation to the skin after prolonged
contact. Some individuals may be more sensitive to exposure. This product contains HCFC
141B which can cause dermatitis.

INGESTION: This is not considered a common occupational route of exposure, and no
observable effects have been demonstrated.

THRESHOLD LIMIT VALUE (ACGIH): No TLV has been established for this product as a
system. It contains HCFC 141B - suggested TWA is 500 PPM.

PERMISSIBLE EXPOSURE LIMIT (OSHA): Same as above.
SECTION VI. EMERGENCY & FIRST AID PROCEDURES

EYE CONTACT: Flush with clean, lukewarm water at low pressure for at least 15 minutes,
occasionally lifting eyelids. Consult a physician immediately.




[image: image11.png]SKIN CONTACT: Remove contaminated clothing. Wash exposed area with warm soapy
water thoroughly. Contaminated clothing should be properly laundered before reusing.

INHALATION: Remove victim from area of exposure to safe area.

INGESTION: Induce vomiting. Never give anything to drink to an unconscious person or
induce vomiting in an unconscious person.

SECTION VIL. SPECIAL PROTECTION INFORMATION

EYE PROTECTION: Liquid chemical goggles or full face shield. No contact lenses should be
worn,

SKIN PROTECTION: Chemical resistant gloves such as natural rubber or polyviny! alcohol.

Cover as much exposed skin as possible with appropriate clothing. If skin creams are used,
keep the area covered by the cream to a minimum.

RESPIRATORY PROTECTION: This product has demonstrated no observable effects at
room temperature, however, it is highly recomended that an air purifying respirator with
organic filter cartridges be worn. In addition, in any spray application, a supplied air source
must be provided.

VENTILATION: Natural or mechanical. Local exhaust will keep the TLV below minimum in
most cases. Spills or other emergencies may require more forceful ventilation means.

OTHER: Safety showers and eye wash stations should be provided in all work areas. All
employees should be properly trained.

SECTION VIII. REACTIVITY DATA
STABILITY: Stable
POLYMERIZATION: Will not occur

INCOMPATIBILITY (MATERIALS TO AVOID): Avoid contact with isocyanates and other
substances that react with hydroxy! groups.

HAZARDOUS DECOMPOSITION PRODUCTS:  Aliphatic fragments, CO, NH3, CO2
SECTION IX. SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS SPILLED OR RELEASED: Contain the

spilled material and then cover with a loose, absorbant material such as oildry, vermiculite,

sawdust or Fuller's earth. Shovel waste material into proper waste containers. Wash the

contaminated areas with hot soapy water thoroughly. Ventilate area to remove vapors.

WASTE DISPOSAL METHODS: Waste material may be incinerated or disposed of under
local, state and federal regulations controlling environmental protection.

SECTION X. SPECIAL PRECAUTIONS/STORAGE DATA

STORAGE TEMPERATURE (min/max): 65° F (18° C) to 75° F (24° C)





[image: image12.png]AVERAGE SHELF LIFE: 6 months from date of manufacture

SPECIAL SENSITIVITY (heat, light, moisture): This product is hygroscopic.
Containerasshould be tightly sealed to prevent moisture contamination. Do not expose to high
temperatures for any length of time.

PRECAUTIONS IN HANDLING/STORAGE: If contamination with isocyanates is
suspected, do not re-seal container because of possible rupture due to pressure buildup.
This product contains HCFC 141B. Exposure to elevated temperatures may cause the 141B
to vaporize in the container czusing a pressure buildup. Always slowly vent container when
opening to relieve any pressure buildup.

SECTION XI. SHIPPING DATA

TECHNICAL SHIPPING NAME: Polyether Polyol Blend
DOT HAZARD CLASSIFICATION: Not-regulated
FREIGHT CLASS (bulk): Polypropylene Glycol
FREIGHT CLASS (package): Polypropylene Glycol
PRODUCT LABEL: "B" Component Polyol

PLACARDS REQUIRED: None

HMIS: F-1, H-1,R-0

THIS INFORMATION 1S FURNISHED WITI{OUT WARRANTY, EXPRESSED OR IMPLIED, EXCEPT
THAT IT IS ACCURATE TO THE BEST KNOWLEDGE OF UCSC. THE DATA ON THESE SHEETS
RELATE ONLY TO THE SPECIFIC MATERIAL DESIGNATED HEREIN. UCSC ASSUMES NO LEGAL
RESPONSIBILITY FOR USE OR RELIANCE UPON THIS DATA.





